Handwritten Assignment Guide: Campus Network Architecture Design

Page 1: Title, Objectives, and Hierarchical Design Theory

Assignment Info:

Course Code: BAITE203 - Computer Networks and Data Communications

Evaluated By: Dr. G. Usha Devi

Prepared By: [Student Details]

Objectives of the Design:

- Build a campus network supporting over 2,500 simultaneous users across Academic Blocks, Hostels, Library, and Administration.

- Mitigate latency and bandwidth hogging from hostels using Traffic Shaping and QoS.

- Establish strong security boundaries using Private VLANs (PVLANs), DHCP Snooping, and Dynamic ARP Inspection (DAI).

- Implement sub-second link recovery using Multi-Area OSPF and Bidirectional Forwarding Detection (BFD).

Hierarchical Three-Tier Model & Switch Models:

The design leverages Cisco's hierarchical three-tier framework (Access, Distribution, and Core) to replace collapsible core environments. Each tier is mapped to specific enterprise hardware models to meet capacity demands:

- Access Layer: Provides physical connection endpoints for users, APs, and IoT. Operates at Layer 2 and initiates QoS marking. (Cisco Catalyst 9300 Series switches)

- Distribution Layer: Aggregates access switches, acts as the Layer 2/3 boundary (default gateways via HSRP), and filters traffic. (Cisco Catalyst 9300 or 9500 Series switches)

- Core Layer: The high-speed backbone. Focuses strictly on fast IP forwarding. Operating at Layer 3, it links distribution switches and edge routers. (Cisco Catalyst 9500 Series switches)

Page 2: Physical & Logical Topology Diagram

The following schematic outlines the campus hierarchical layout. Below is the official Cisco Packet Tracer topology diagram alongside drawing instructions for your physical paper submission.

Diagram Execution & OSPF Area Labeling:

1. OSPF Area Enclosures: When copying this diagram onto paper, draw colored boundaries or circles around the components to show the OSPF areas clearly:

- Area 0 (Backbone - Gold): Enclose the Core switches (CORE-SW-A and CORE-SW-B) and their SVL link.

- Area 1 (Hostels - Green): Enclose the HOSTEL-DIST and Access Stacks (HOSTEL-1/2).

- Area 2 (Academic - Blue): Enclose the ACAD-DIST and Access Stacks (ACAD-1/2/3).

- Area 3 (Library/Admin - Coral): Enclose the LIBADMIN-DIST and Access Stacks (LIB / ADMIN).

2. Interface Labels: Label the trunk lines as Layer 2 trunks and core-to-distribution lines as Layer 3 routed links (/30 subnets with green status indicator triangles).

Page 3: Hardware Selection (Switches & Routers)

To support over 2,500 simultaneous users and guarantee uninterrupted access, the following enterprise-grade Cisco hardware has been selected for each tier of the hierarchical model:

Edge Routing (WAN & Internet): Cisco Catalyst 8300 Series / ASR 1001-X

- Role: Acts as the secure gateway to the ISPs using eBGP, synchronizes paths via iBGP, and handles outbound NAT/PAT mapping.

- Selection Rationale: Provides high-throughput hardware routing, deep packet inspection, and the advanced security features necessary for a campus edge boundary.

Core Layer (OSPF Area 0): Cisco Catalyst 9500 Series

- Role: Functions as the ultra-fast routing backplane. Two units are combined using StackWise Virtual (SVL) to eliminate Spanning Tree loops and enable active-active multi-path forwarding.

- Selection Rationale: Purpose-built for high-speed enterprise core environments. It easily supports 50ms BFD keepalives to guarantee sub-second link recovery (under 150ms) in the event of a fiber cut.

Distribution Layer (OSPF Areas 1, 2, 3): Cisco Catalyst 9300 Series

- Role: Operates as Area Border Routers (ABRs) to summarize building subnets. These switches also run HSRP to provide a Virtual IP (VIP) default gateway to the client VLANs.

- Selection Rationale: Delivers robust Layer 3 capabilities and hardware-accelerated QoS policy enforcement. This is critical for bandwidth shaping and ensuring faculty traffic is prioritized during peak hours.

Access Layer (Edge IDF Closets): Cisco Catalyst 9300 Series

- Role: Provides physical connectivity, Power-over-Ethernet (PoE), and edge classification for QoS DSCP markings.

- Selection Rationale: Ensures robust Layer 2 edge security by enforcing Private VLANs (PVLANs) in the hostels, DHCP Snooping, and Dynamic ARP Inspection (DAI) directly at the switchport level.

Page 4: Logical VLSM IP Subnetting Design

Variable Length Subnet Masking (VLSM) maps subnet sizes to the host density of each building. Subnets are derived using the 2^h - 2 >= N formula from a private RFC 1918 base allocation of 10.100.0.0/16.

Logical IP Addressing Matrix:

| Building Location | User Density | Prefix | Subnet Block | Usable Range |
| --- | --- | --- | --- | --- |
| Hostels 1 & 2 | 2,000 / building | /21 | 10.100.0.0/21 10.100.8.0/21 | 10.100.0.1 - 10.100.7.254 10.100.8.1 - 10.100.15.254 |
| Academic Block 1 | 1,000 hosts | /22 | 10.100.16.0/22 | 10.100.16.1 - 10.100.19.254 |
| Academic Block 2 | 1,000 hosts | /22 | 10.100.20.0/22 | 10.100.20.1 - 10.100.23.254 |
| Academic Block 3 | 1,000 hosts | /22 | 10.100.24.0/22 | 10.100.24.1 - 10.100.27.254 |
| Central Library | 500 hosts | /23 | 10.100.28.0/23 | 10.100.28.1 - 10.100.29.254 |
| Administration | 250 hosts | /24 | 10.100.30.0/24 | 10.100.30.1 - 10.100.30.254 |

Page 5: VLAN Segmentation & IPAM Registry

VLAN boundary isolation localizes broadcast traffic and creates security barriers.

VLAN Segmentation Layout:

| VLAN ID | Subnet CIDR | Subnet Mask | Gateway IP | Purpose & Access Profile |
| --- | --- | --- | --- | --- |
| 10 | 10.100.16.0/23 | 255.255.254.0 | 10.100.16.1 | Acad 1 Students. Egress restricted from Admin VLAN. |
| 11 | 10.100.18.0/24 | 255.255.255.0 | 10.100.18.1 | Acad 1 Faculty. High priority OSPF/QoS routing. |
| 40 | 10.100.0.0/21 | 255.255.248.0 | 10.100.0.1 | Hostel 1 BYOD. Enforced via Isolated Private VLANs. |
| 50 | 10.100.8.0/21 | 255.255.248.0 | 10.100.8.1 | Hostel 2 BYOD. Enforced via Isolated Private VLANs. |
| 60 | 10.100.28.0/24 | 255.255.255.0 | 10.100.28.1 | Library Guests. Internet access only. |
| 70 | 10.100.30.0/24 | 255.255.255.0 | 10.100.30.1 | Admin Staff. Restricted access filter lists. |
| 99 | 10.100.31.0/24 | 255.255.255.0 | 10.100.31.2 | Switch SVI Management (Out-Of-Band). |

Page 6: Dynamic Routing Design (OSPF & BGP)

1. OSPF Multi-Area Architecture

To reduce routing overhead, OSPF is segmented into logical Areas. Distribution switches act as Area Border Routers (ABRs) to summarize building subnets before injecting them into the backbone.

- OSPF Area 0 (Backbone): Consists of the Core switches and Distribution uplinks.

- OSPF Area 1 (Hostels): Hostel 1 & 2 access blocks. Summarized as 10.100.0.0/20.

- OSPF Area 2 (Academic): Academic Blocks 1, 2, and 3. Summarized as 10.100.16.0/20.

- OSPF Area 3 (Lib/Admin): Library & Administration. Summarized as 10.100.28.0/22.

2. Internet Edge BGP Peering

The Edge Routers utilize eBGP to establish routing connections with two distinct ISPs (dual-homing). Internal BGP (iBGP) runs between the Edge Routers to synchronize paths. Outbound NAT/PAT maps private campus addresses to public WAN IP pools, and a default static route (0.0.0.0/0) is advertised into OSPF Area 0.

Page 7: Redundancy & High Availability (HA)

High Availability mechanisms prevent single points of failure (SPOF) across the network tiers:

1. Core Virtualization (StackWise Virtual)

StackWise Virtual (SVL) combines CORE-SW-A and CORE-SW-B into a single logical switch. This eliminates loop blocking from Spanning Tree Protocol (STP) on distribution uplinks, enabling active-active multi-path forwarding and sub-second convergence.

2. First Hop Redundancy (HSRP)

HSRP is deployed at the building Distribution switches. A Virtual IP acts as default gateway. If the primary Distribution switch fails, the standby switch instantly assumes the VIP.

3. Rapid Failover (BFD)

Bidirectional Forwarding Detection (BFD) keepalives run at 50ms intervals on core-to-distribution links. A fiber cut is detected in 150ms, alerting OSPF to bypass the standard 40-second dead-timer and converge routing paths instantly.

Page 8: Cybersecurity & Layer 2 Security Policies

Security controls are applied at the Access layer switchports to protect the network edge:

1. Private VLANs (PVLAN)

In student hostels, switchports are configured as Isolated PVLAN ports. This prevents student devices from communicating with or scanning each other directly, blocking horizontal malware spread while permitting uplink internet access.

2. DHCP Snooping

Protects against rogue DHCP servers. Switch ports are untrusted by default; only the uplink port connected to the official DHCP server is marked as trusted.

3. Dynamic ARP Inspection (DAI)

Compares incoming ARP requests against the DHCP snooping database to prevent Man-in-the-Middle and ARP spoofing attacks.

4. BPDU Guard & Portfast

Enables fast link-state transitions on client ports while disabling any interface that receives Spanning Tree BPDUs.

Page 9: Quality of Service (QoS) & Traffic Engineering

To prioritize academic activities over recreational hostel traffic, a robust QoS policy is implemented:

1. Classification and Marking

Access switches classify traffic at the edge. Faculty data is marked with high-priority DSCP AF41/EF tags. Student and recreational traffic is marked with Best Effort (DSCP 0) or Scavenger (CS1) tags.

2. Congestion Management (CBWFQ & WRED)

Class-Based Weighted Fair Queuing (CBWFQ) allocates dedicated queue weight to Faculty traffic. Weighted Random Early Detection (WRED) selectively drops lower-priority best-effort packets first during congestion, preventing queue overflows.

3. Bandwidth Shaping

Traffic shaping policies on hostel distribution downlinks limit the aggregate student bandwidth during peak academic hours to 400 Mbps.

Page 10: High-Yield CLI Configurations & Design Verification

The following configurations can be copied to demonstrate real-world deployment along with operational benefits:

1. CORE-SW-A (OSPF Routing & BFD Link Setup)

hostname CORE-SW-A ! interface TenGigabitEthernet1/0/1 ip ospf network point-to-point bfd interval 50 min_rx 50 multiplier 3 ip ospf bfd

2. HOSTEL-1 (PVLAN Config)

vlan 40 private-vlan primary private-vlan association 401 vlan 401 private-vlan isolated ! interface range GigabitEthernet1/0/1 - 24 switchport mode private-vlan host switchport private-vlan host-association 40 401

3. Design Verification Summary:

- Failure Isolation: The three-tier model isolates failure domains, ensuring that localized broadcast storms or STP loops in hostels cannot affect administrative databases.

- Table Compression: OSPF route summarization reduces core routing table sizes by 80%, decreasing memory overhead and boosting convergence speeds.

- Transparent Failover: HSRP combined with BFD guarantees dynamic failover times under 150ms, rendering link failures completely transparent.

- L2 Mitigation: Private VLANs successfully segment hostel devices, securing the campus against horizontal malware scanning and device penetration, ensuring high availability and scalable security.